✅ PAUSE / RESUME & M600 / RUNOUT VALIDATION CHECKLIST

DGUS UI + Klipper State Machine + Runout Sensor Integration

🟦 SECTION A — PRE-TEST SETUP

☐ A1. Printer powered on and idle

☐ A2. DGUS UI responsive

☐ A3. Filament loaded

☐ A4. Runout sensor type confirmed

- Switch sensor

- Motion sensor

- No sensor

☐ A5. Confirm purge-line protection is active

- PAUSE and STOP buttons disabled until purge line begins

- Attempting to press PAUSE/STOP before purge line should do nothing

🟦 SECTION B — START PRINT & PURGE LINE

☐ B1. Start a print

☐ B2. Observe purge line

☐ B3. Confirm PAUSE/STOP become enabled only after purge line completes

☐ B4. Confirm UI shows PRINTING

☐ B5. Confirm timers behave correctly

- Elapsed time increments continuously

- Printing time increments continuously

🟦 SECTION C — MANUAL PAUSE / RESUME

C1. Press PAUSE button

☐ Toolhead parks ☐ pause_resume.is_paused == True ☐ print_stats.state becomes "paused" within 1–2 seconds ☐ UI switches to PRINT PAUSED within 2 seconds ☐ Elapsed time continues ☐ Printing time freezes

C2. Press RESUME button

☐ Toolhead returns to print ☐ pause_resume.is_paused == False ☐ print_stats.state == "printing" ☐ UI switches to PRINTING ☐ Printing time resumes from frozen value ☐ Elapsed time continues uninterrupted

🟦 SECTION D — FILAMENT RUNOUT (M600 TRIGGERED)

D1. Cut filament during printing

☐ Runout sensor triggers ☐ M600 executes ☐ Toolhead parks ☐ pause_resume.is_paused == True ☐ filament_present == False ☐ UI switches to PRINT PAUSED within 2 seconds ☐ Elapsed time continues ☐ Printing time freezes

D2. Press RESUME without inserting filament

☐ Klipper prints: “Resume aborted — no filament” ☐ Toolhead remains parked ☐ print_stats.state == "paused" ☐ UI remains on PRINT PAUSED ☐ No flicker to PRINTING ☐ No CLEAR_PAUSE side effects

D3. Insert filament

☐ filament_present == True ☐ UI stays on PRINT PAUSED ☐ No automatic resume

D4. Press RESUME after inserting filament

☐ Toolhead returns to print ☐ pause_resume.is_paused == False ☐ print_stats.state == "printing" ☐ UI switches to PRINTING ☐ Printing time resumes from frozen value

🟦 SECTION E — CANCEL / STOP BEHAVIOR

E1. Cancel print while printing

☐ Press STOP ☐ Print aborts cleanly ☐ UI switches to PRINT FINISHED screen ☐ No stale pause state ☐ No CLEAR_PAUSE needed

E2. Cancel print while paused

☐ Pause print ☐ Press STOP ☐ UI switches to PRINT FINISHED ☐ No flicker ☐ No false resume

E3. Cancel print during runout

☐ Trigger runout ☐ Press STOP ☐ UI switches to PRINT FINISHED ☐ No false transitions

🟦 SECTION F — SENSOR EDGE CASES

F1. Disable sensor in config

☐ Set sensor enabled: False ☐ Start print ☐ No false runout ☐ PAUSE/RESUME behave normally

F2. No sensor installed

☐ Remove sensor config ☐ Start print ☐ No false runout ☐ PAUSE/RESUME behave normally

F3. Motion sensor slow-movement test

☐ Extrude at very slow speed (0.5 mm/s) ☐ Confirm no false runout

🟦 SECTION G — POWER CYCLE RECOVERY

G1. Pause print, then reboot printer

☐ Pause print ☐ Power cycle ☐ Start new print ☐ No stale pause state ☐ No CLEAR_PAUSE needed ☐ UI starts cleanly on PRINTING

🟦 SECTION H — FINAL VALIDATION

☐ H1. All transitions between PRINTING ↔ PRINT PAUSED behave correctly

☐ H2. No flicker or false transitions

☐ H3. No incorrect CLEAR_PAUSE behavior

☐ H4. Printing time freezes on pause and resumes correctly

☐ H5. Elapsed time continues uninterrupted

☐ H6. PRINT FINISHED screen appears on STOP/CANCEL

☐ H7. Runout behavior is stable and predictable

☐ H8. RESUME aborts correctly when filament is missing

☐ H9. UI never switches to PRINTING during a failed RESUME

⭐ If all boxes are checked…

You have a fully validated, race-condition-proof, sensor-agnostic, time-accurate, UI-synchronized pause/resume system.

📝 TEST FAILURES LOG

(Designed for clipboard use beside the printer)

QA FAILURE LOG — PAUSE / RESUME / M600 VALIDATION

Tester: ____________________ Date: ______________________

Firmware Version: ____________________ DGUS Version: ________________________

Runout Sensor Type: ☐ Switch ☐ Motion ☐ None

Print File Used: _______________________________________

Failure Entry #____

Step ID: ________ (e.g., C1, D2, E3)

Expected Behavior:

Actual Behavior:

Severity: ☐ Critical (unsafe motion, heater fault, MCU crash) ☐ Major (UI desync, incorrect state, RESUME failure) ☐ Minor (cosmetic, timing, non-blocking)

Reproducibility: ☐ Always ☐ Intermittent ☐ Rare ☐ One-off

Environment Notes:

- Sensor enabled: ☐ Yes ☐ No

- Filament present at start: ☐ Yes ☐ No

Additional Notes / Hypothesis:

Continue Testing? ☐ Yes ☐ No — Reason: ___________________________________________

(Print multiple copies of the failure entry block — QA engineers often fill 5–20 per session.)